******************************************************************************

Bitmap Graphics Functions

******************************************************************************

A bit map uses one VDP2 scroll screen for graphics display.  Only the 256

color palette mode is supported by SGL.

After normal initialization processing is completed, the bitmap library can be

initialized by executing slInitBitMap().

******************************************************************************

Bool slInitBitMap((Uint16)scrn, (Uint16)size, (void*)adr)

******************************************************************************

Function:

Sets the bit map screen to the 256-color mode. The screen is not cleared.

scrn:

Sets either bmNBG0, bmNBG1, or bmRBG0.

size:

Sets either BM_512x256, BM_512x512, BM_1024x256, or BM_1024x512.

adr:

Sets the start address of the VRAM area used as the bit map.

This area must be within a 20000H boundary. In this function,

slBitMapNbg0(Nbg1, Rbg0) is executed. The center point of the screen is set as

the origin.

******************************************************************************

void slBitMapBase((Sint16)xbase, (Sint16)ybase)

******************************************************************************

Function:

Sets the origin point of the bit map screen.

Specifies the upper left of the screen as (0, 0).

******************************************************************************

Sint8  *slLocateBitMap((Sint16)xps, (Sint16)yps)

******************************************************************************

Function:

Returns the VRAM address of the specified position.

When the position is outside the screen, NULL is returned.

******************************************************************************

void slClearBitMap((Sint8)color);

******************************************************************************

Function:

Clears the bit map screen with the specified color.

******************************************************************************

Bool slBMPset((Sint16)xps, (Sint16)yps, (Sint8)color)

******************************************************************************

Function:

Draws a pixel at the specified position. When the position is outside the

screen, FALSE is returned.

******************************************************************************

Sint8 slBMPoint((Sint16)xps, (Sint16)yps)

******************************************************************************

Function:

Reads the pixel at the specified position and returns its palette code.

When the coordinate is outside the screen, 0 is returned.

******************************************************************************

void slBMCircle((Sint16)xps, (Sint16)yps, (Sint16)radius, (Sint8)color)

******************************************************************************

Function:

Draws a circle with the specified position as its center.

******************************************************************************

Bool slBMLine((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8)color)

******************************************************************************

Function:

Draws a straight line which connects two specified points.

When the position is outside the screen, FALSE is returned.

******************************************************************************

Bool slBMBox((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8)color)

******************************************************************************

Function:

Draws a rectangle with the specified points as its vertices.

When the points are outside the screen, FALSE is returned.

******************************************************************************

Bool slBMBoxFill((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8)color)

******************************************************************************

Function:

Draws a rectangle with specified points as its vertices.

The interior of the rectangle is also filled. When the points are outside the

screen, FALSE is returned.

******************************************************************************

Bool slBMGet((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8*)ptr)

******************************************************************************

Function:

Copies bit map data into the buffer for a rectangle with two specified points

as its vertices. If a part of the rectangle is outside of the screen, the data

for that portion is not copied. When the entire rectangle is outside the

screen, FALSE is returned.

******************************************************************************

Bool slBMPut((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8*)ptr)

******************************************************************************

Function:

Copies bit map data from the buffer into a rectangle with two specified points

as its vertices. If a part of the rectangle is outside of the screen, the data

for that portion is not copied. When the entire rectangle is outside the

screen, FALSE is returned.

******************************************************************************

Bool slBMSprPut((Sint16)xps0, (Sint16)yps0, (Sint16)xps1, (Sint16)yps1,

(Sint8*)ptr)

******************************************************************************

Function:

Copies bit map data from the buffer into a rectangle with two specified points

as its vertices. However, spaces (palette code 0) are not copied. If part of

the rectangle is outside the screen, that portion is not copied. When the

entire rectangle is outside the screen, FALSE is returned.